FM Receiver with Si4703 DSP & RDS Decoding

## Team members

- Mezera Vojtěch

- Moravec David

- Mostecký Filip

- Pavlíček Michal

Git hub

## Analog Keypad Control

- S1   – next station / increase frequency

- S2   – previous station / decrease frequency

- S3   – volume down

- S4   – volume up

- S5   – toggle tuning mode (AUT ↔ MAN)

## Radio Functions

### Automatic Mode (AUT)

- Default at start

- Scans FM band, evaluates signal via si4703_getRSSI()

- Threshold: 20 (0–127); stations ≥19 saved to list

- S1/S2 cycle through list (wrap-around enabled)

### Manual Mode (MAN)

- Activated by S5

- S1: +0.1 MHz, S2: –0.1 MHz

### Volume

- S3: down, S4: up

- Value shown on OLED display

### RDS

- Displays station text if available

- Updates dynamically

- Shows last valid text or if unavailable -----

Firmware Architecture

Event-Driven Design & Multitasking The firmware operates on a non-blocking main loop principle to ensure system responsiveness.

Auto-Scan Algorithm

Explains the logic in automaticke_ladeni function in main.c.

Signal Quality Evaluation Upon startup, the system performs an automatic bandwidth scan (87.5 MHz – 108.0 MHz):

- The tuner seeks the next available carrier frequency.

- The microcontroller reads the RSSI (Received Signal Strength Indicator) from the Si4703 registers.

- Only stations exceeding the defined signal threshold (RSSI > 20) are stored in the RAM station list.

;

Abstract

This project implements a robust FM radio receiver utilizing the Si4703 DSP tuner and ATmega328P microcontroller. Key features include automatic station seeking with signal quality evaluation (RSSI), a memory system for 20 stations, and non-blocking RDS decoding for real-time text display without user interface latency. Inputs are handled via an ADC resistor-ladder keyboard to conserve GPIO pins.