# FM radio receiver with digital tuning

Git hub

Abstract

The main objective of this project is to implement fully functional FM radio receiver with digital tuning on ATmega328P-based Arduino Uno board. The tuner module Si4703 is controlled over the I2C bus. I2C OLED display is used for the main user interface. Basic push buttons are used for controlling the receiver. This project also features the RDS (Radio Data System) to show the station name and radio text.

## Team members

- Mezera Vojtěch

- Moravec David

- Mostecký Filip

- Pavlíček Michal

## Button Control

- S1 – next station / increase frequency

- S2 – previous station / decrease frequency

- S3 – volume down

- S4 – volume up

- S5 – toggle tuning mode (AUT ↔ MAN)

## Radio Functions

### Automatic Mode (AUT)

- Default at start

- Scans FM band, evaluates signal via si4703_getRSSI()

- Threshold: 19 (0–127); stations ≥19 saved to list

- S1/S2 cycle through list (wrap-around enabled)

### Manual Mode (MAN)

- Activated by S5

- S1: +0.1 MHz, S2: –0.1 MHz

### Volume

- S3: down, S4: up

- Value shown on OLED display

### RDS

- Displays station text if available

- Updates dynamically

- Shows last valid text or if unavailable -----